Chapter 8: Image Compression

1.Introduction to Image Compression

Image compression refers to a set of techniques used to reduce the amount of digital data needed to store or transmit an image. The main objective is to minimize data size while maintaining acceptable visual quality and preserving essential information.

2.Importance of Image Compression

Without compression, digital images require large storage space and long transmission times. Compression helps to:

- Save memory and storage devices

- Speed up image transfer over networks

- Reduce bandwidth consumption

3.Difference Between Data and Information

- Data represents the raw bits used to encode an image

- Information represents the meaningful visual content perceived by humans

The same information can be stored using different amounts of data, which is why compression is possible.

4.Areas of Application

Image compression is widely used in:

- Internet and multimedia systems

- Digital cameras and smartphones

- Video calls and online meetings

5. Redundancy in Digital Images

Compression techniques aim to remove redundancy, which refers to unnecessary or repetitive data.

5.1 Coding Redundancy

Coding redundancy occurs when symbols are represented using more bits than required.

Efficient coding assigns fewer bits to frequently occurring symbols and more bits to rare symbols.

Average Code Length Equation:

L_avg = Σ (p_i × l_i)

Where:

- p_i is the probability of symbol i

- l_i is the code length of symbol i

Reducing L_avg improves compression efficiency.

5.2 Interpixel Redundancy

Pixels in an image are not independent; neighboring pixels often have similar intensity values. This correlation allows prediction and removal of repeated information.

5.3 Psychovisual Redundancy

The human visual system does not perceive all image details equally. Fine details and high-frequency components are less noticeable, allowing them to be reduced or removed without significantly affecting perceived quality.

6. Classification of Image Compression Methods

6.1 Lossless Compression

- Preserves all image information

- Original image can be exactly reconstructed

Common lossless methods include:

- Huffman Coding

- Run-Length Encoding (RLE)

6.2 Lossy Compression

- Allows controlled loss of information

- Achieves higher compression ratios

- Image quality is slightly degraded

Lossy compression is suitable for natural images where perfect reconstruction is not required.

7. Huffman Coding

- Statistical method to reduce redundancy.

- Steps: calculate symbol probabilities → combine least probable symbols → build tree → assign shorter codes to frequent symbols.

- Average code length: Lavg=∑(pi×li)

- Prefix-free codes ensure error-free decoding.

8. Run-Length Encoding (RLE)

- Replaces repeated symbols with (symbol, count).

- Efficient for images with large uniform areas, poor for random data.

9. Entropy

- Theoretical lower bound of compression: H=-∑pilog⁡2pi

- No lossless method can compress below entropy.

10. Compression Ratio

- Compression ratio = Original Size ÷ Compressed Size

- Higher ratio → better compression.